PreciseFlex 400 Horizontal Force Compliance

Version 1.0A4, June 26, 2014

“HorizCompliance” is a software package for the PreciseFlex 400 Sample Handlers that permits these robots to automatically correct horizontal positioning errors when an object, such as a sample plate, is being grasped or placed.  It does this by permitting the axes that control horizontal movement to “float” or “comply” to horizontal reaction forces while other axes are being driven.  When grasping, the horizontal axes will float as the fingers close around a plate.  When placing a plate, the horizontal axes will quickly and automatically slide over as a plate contacts the chamfers of the nest into which the plate is placed.

The PreciseFlex 400 Sample Handlers are ideally suited for this task due to their geometry, and low-friction back-drivable axes.  These features permit this robot to react very quickly and with minimal reaction forces.  In typical applications, enabling horizontal compliance mode has very little impact on cycle time.  At the same time, compliance mode can significantly improve the reliability of operations by eliminating problems such as plates springing out of their nests when they are released due to residual forces caused by placement errors.

In contrast, 6-axis articulated robots typically are very heavy, have high friction axes, and are equipped with very high gear ratio motor drives that are difficult to back-drive.  These features make it extremely difficult to automatically and quickly comply to small contact forces.  In fact, with this type of robot, it is often very difficult to delicately position objects without generating very high contact forces.

Using this compliance function is very straightforward.  Two GPL routines, “AC_HorizCompliance” and “AC_HorizCompliance2”, are provided for enabling and disabling compliance mode.  The second routine is very similar to the first routine, except that the second routine permits optional bias torques to be applied to each of the robot’s horizontal axes to compensate for small offsets in free floating torques for these axes.  These routines are part of a larger force based software package that permits the Sample Handler to automatically teach plate nest locations.

AC_HorizCompliance is integrated with the TCP Command Server PF400 plug-in and can be executed as part of the “pickplate” and “placeplate” commands.  Alternatively, either of the routines can be installed as part of a customer specific GPL software project.

### 1.   System Requirements

The requirements for the execution of this package are as follows:

- Any model of the PreciseFlex 400 family of Sample Handlers with various lengths of Z-travels, ranges of reaches, and grippers and fingers.

- A copy of the TCP Command Server Version 2.0 or later that includes the Auto Center plug-in module.

- GPL 3.2E8 or later installed on the controller for the Sample Handler.

- A “Precise Application” or “XY Compliance” License installed on the controller for the Sample Handler.

### 2.   Executing AC_HorizCompliance Within the TCP Command Server

Execution of the AC_HorizCompliance routine is built into the “pickplate” and “placeplate” commands within the PF400 plug-in.  If specified as part of these commands, the AC_HorizCompliance routine is automatically executed.  Please see the documentation for these commands for information on specifying compliance mode.

### 3.  Installing AC_HorizCompliance/2 In A Custom GPL Application

AC_HorizCompliance/2 can be executed from within the TCP Command Server project in which they are distributed or they can be included in a customer specific application.  To include these functions within your own custom GPL project, do the following:

- Launch the Guidance Development Environment.

- In the Project Manager window, right click on your GPL project.

- Select “Add new item”.

- In the Add New Item pop-up, click on the “Code File” icon, fill in the Name field with “Pf400_auto_center.gpp” and press Add.  This will add a dummy placeholder code file into your project.

- Using Windows Explorer, locate your copy of the TCP Command Server project and copy the Pf400_auto_center.gpp” file to your own project folder.  Replace the dummy placeholder file with the real code file.

The HorizCompliance function can now be executed from within your custom application.

### 3.1   Execution

The HorizCompliance function consists of three top-level callable GPL routines:

AC_ErrorMessage  -  This routine converts both standard system exceptions and custom exceptions generated by HorizCompliance into a String for output to the operator.

AC_ HorizCompliance   -  This is the main entry point for the horizontal compliance function.  It both enables and disables compliance mode.

AC_ HorizCompliance2   -  This is a secondary entry point for the horizontal compliance function that permits an optional bias torque to be applied to each horizontal axis.  It both enables and disables compliance mode.

The calling sequence for each of these routines is provided in Appendix A.

This concludes the description of the HorizCompliance function.

Appendix A:  GPL Routine Calling Sequences

Public Function AC_ErrorMessage(Exc1 As Exception) As String

' ABSTRACT: This is a TOP-LEVEL CALLABLE ROUTINE that converts an Exception

'		object to a string message.  The message will be of the form:

'

'				-error_code, *Error message*

'

'		If the error is not a standard system exception but one generated by

'		this application, it's error code will range from -2000 to -2999.

'		For such errors, this function returns an application specific

'		error message.

'

' INPUTS:	Exc1		Exception to be converted to a string

'

' OUTPUTS:	None

'

' RETURNS:	String that contains the converted error message.

Public Function AC_HorizCompliance(Rb As Integer, Enable_Comp As Boolean) _

As Exception

' ABSTRACT: This is a TOP-LEVEL CALLABLE ROUTINE that enables and

'		disables compliance in the horizontal plane for the PF400

'		robot at the end of the currently executing motion.

'

'		Since compliance mode changes the operating mode of selected

'		axes of the robot, enabling and disabling this mode will break

'		continuous path motions.

'

'		If compliance mode is not turned on, disabling compliance mode

'		performs no operation.

'

' INPUTS:	Rb				Robot number (1-N_ROB).

'			Enable_Comp		True to enable compliance, False to disable.

'

' OUTPUTS:	None

'

' RETURNS:  Exception that indicates if any error has occurred.  The most

'		common error is that the license is not installed to permit

'		compliance mode in the PF400 robot.

Public Function AC_HorizCompliance2(Rb As Integer, Enable_Comp _

As Boolean, Reduced_Pct As Double) As Exception

' ABSTRACT: This is a TOP-LEVEL CALLABLE ROUTINE that enables and

'       disables compliance in the horizontal plane for the PF400

'       robot at the end of the currently executing motion.

'

'		This performs the same function as AC_HorizCompliance but

'		includes an additional parameter to permit a bias torque to be

'		set for each horizontal axis that is equal to a percentage of the

'		compensator output for these axes just before compliance mode

'		is enabled.  This was added because axes sometimes need a small

'		holding torque to maintain position and turning off all torque

'		resulted in axes twitching into a neutral position.

'

'       Since compliance mode sets axes of the robot into torque

'       control mode, enabling and disabling torque control mode

'       will break continuous path motions.

'

'       If compliance mode is not turned on, disabling compliance mode

'       is a NOOP.

'

' INPUTS:   Rb              Robot number (1-N_ROB).

'           Enable_Comp     True to enable compliance, False to disable.

'           Reduced_Pct     Reduced torque to this percentage of the last

'                           used torque in position control mode, 0-100.

'

' OUTPUTS:  None

'

' RETURNS:  Exception that indicates if any error has occurred.  The most

'       common error is that the license is not installed to permit

'       compliance in the PF400 robot.